HEC-RAS Run Sheet - Wadi Majlas Dam Break Analysis

Companion to the Dam Break Analysis Methodology Report Rev 00 (September 2026). Values only; the reasoning is in Rev 00, chapter given in brackets. Units: m, m a.s.l., m³/s, h. Project CRS EPSG:32640. HEC-RAS 6.6, SI units. Levels: wadi bed 23.0, FSL 84.5, crest 95.5, parapet 96.7, PMF pool 96.7.

## What to write down

Table 1 - Run register.

| Run register column | Entry |
| --- | --- |
| Run ID | S1-D, S1-N, S2-D, S2-N, S3-D, S3-N; sensitivity SW1, SW2, SW3, ST1, ST2, SC1, SN1, SN2, SM1, SS1, SP1, SF1, SI1, SR1, SPW1 |
| HEC-RAS files | Project Majlas_DamBreak.prj; plan = Run ID; geometry G_Dikes or G_NoDikes; unsteady U_S1 (no inflow), U_PMF, U_10000; terrain T_Dikes or T_NoDikes |
| Inputs | Start pool; inflow file; breach width, bottom, time, coefficient, trigger; sea level; dikes; mesh version; Manning table version |
| Run | Date, modeller, HEC-RAS version, run time, restart file used |
| Checks | Volume error %; max Courant; cells with iteration warnings; peak within hand-check bounds (Y/N); reviewer initials |

Table 2 - Results to record.

| Value to record from every run | Where |
| --- | --- |
| Breach peak outflow and its time; breach volume | Connection hydrograph (10 s output) |
| Reservoir level at breach start, at peak outflow, 1 h and 6 h after | Reservoir 2D area stage at the dam |
| Peak flow, peak level, arrival time (depth 0.3 m), time to peak | Reference lines: gorge exit, town entry, coast road, shoreline |
| Maximum depth, velocity, D×V at 10 named points in the town | RAS Mapper point values |
| Inundated area above 0.3 m (km²); people in it; plots in it | Max depth raster > 0.3 m, overlay GHS-POP and land-use |
| Volume accounting error; max Courant; iteration warnings; run time | Computation log, HDF summary |
| Exported rasters (2 m GeoTIFF) | Max depth, max velocity, max D×V, arrival time 0.3 m, duration > 0.3 m, hazard AIDR; incremental depth for S2 (S2 minus S3) |

## Model set-up

Table 3 - Geometry.

| Step | Do this | Value |
| --- | --- | --- |
| Terrain | RAS Mapper > Terrains > New; stack in this order, top has priority | 1 dam-site survey 5 cm; 2 design surfaces (dam, spillway, basin, chute, dikes, crossings, roads); 3 drone DTM 2 m; 4 NSA 5 m (existing DS model); 5 NSA 2 m |
| Two terrains | Repeat without the dike surfaces | T_Dikes; T_NoDikes |
| Dam in terrain | Cut the dam body out along the axis (terrain modification, channel) so the connection alone represents it | Bed at axis 23.0 m |
| Land cover | Map Layers > Land Cover from the land-use layer; Manning table | See table below |
| Reservoir 2D area | New 2D area from the dam to the upstream end of the 96.7 m pool plus 200 m margin | Cells 30 to 50 m; 10 m in the last 1 km; check volume at FSL = 102.2 Mm³ ± 2 % |
| Downstream 2D area | Reuse 'DS Protection' from the flood-protection model; extend to the shoreline; add the sea strip | Cells 10 to 15 m plain; 5 m dam and basin; 10 m town; 20 to 30 m coast |
| Breaklines | Dikes (both crests), wadi banks, road embankments, coast road, dam axis, basin walls | Cell size along breakline = zone value; enforce |
| Refinement regions | Dam and basin 5 m; town 10 m | - |
| Reference lines | Gorge exit, town entry, coast road, shoreline | Named as listed |
| Boundary condition lines | Reservoir inflow (upstream end); tributary inflows at HEC-HMS junctions J10, J11, J12; sea (stage); open land edges (normal depth) | - |

Table 4 - Manning's n.

| Land cover | n base | n low | n high |
| --- | --- | --- | --- |
| Wadi bed, gravel | 0.035 | 0.030 | 0.045 |
| Floodplain, bare or scrub | 0.040 | 0.035 | 0.050 |
| Agriculture, date gardens | 0.080 | 0.060 | 0.100 |
| Residential and commercial blocks | 0.120 | 0.080 | 0.200 |
| Roads, paved | 0.018 | 0.016 | 0.022 |
| Concrete (spillway, basin) | 0.015 | 0.013 | 0.017 |
| Riprap on dikes | 0.035 | 0.030 | 0.040 |
| Reservoir bed | 0.035 | 0.030 | 0.045 |
| Sea, lagoon | 0.025 | 0.020 | 0.030 |

Table 5 - Dam connection.

| SA/2D connection field | Entry |
| --- | --- |
| Alignment | Straight dam axis, 312 m, drawn from the design CAD (indicative shapefile in 04_data/gis until the CAD arrives) |
| Weir profile (station, elevation) | Left non-overflow blocks: 95.5; spillway section 212 m gross: 5 bays × 40 m at 84.5 with 4 piers × 3 m at 96.7; right non-overflow blocks: 95.5. Block stations from the joint layout when received; until then left 0-50 m, spillway 50-262 m, right 262-312 m |
| Weir width | 6 m |
| Weir shape, non-overflow blocks | Broad crested; coefficient 1.66 (SI) |
| Weir shape, spillway bays | Ogee; spillway approach height 61.5 m; design energy head from the spillway design (request); resulting C about 2.1 to 2.2 (SI) |
| Culverts, gates | None (bottom outlet closed) |
| Check | No-failure run S3-D: discharge at pool 96.7 m = 17,000 m³/s ± 5 % |

Table 6 - Breach entries.

| Plan > Breach (plan data) | Base | Lower | Upper | Other sensitivity |
| --- | --- | --- | --- | --- |
| Breach this structure | On (S1, S2) | On | On | Off in S3 |
| Breach method | User entered data | - | - | - |
| Center station | Centre of the spillway section | - | - | - |
| Final bottom width (m) | 86 | 43 | 156 | 129 (three bays) |
| Final bottom elevation (m) | 23.0 | - | - | - |
| Left and right side slopes (H:V) | 0 | 0 | 0 | - |
| Breach weir coefficient (SI) | 1.66 | 1.44 | 1.66 | - |
| Full formation time (h) | 0.1 | 0.3 | 0.05 | - |
| Failure mode | Overtopping | - | - | - |
| Trigger, S1 (sunny day) | Set time = simulation start + 2 h | - | - | - |
| Trigger, S2 (flood day) | WS elevation = 96.7 m | - | - | SF1: set time = time of max pool in S3-D + 2 h |
| Breach progression | Linear | - | - | - |

## Scenarios and water levels

Table 7 - Base runs.

| Run | Geometry / terrain | Reservoir initial WSE (m) | Inflow BC | Tributaries J10-J12 | Breach | Sea stage | Window |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S1-D | G_Dikes / T_Dikes | 84.5 | None | None | On; set time +2 h; 86 m; 0.1 h | HAT, constant | 0 to 14 h; warm-up 2 h; restart written at 2 h |
| S1-N | G_NoDikes / T_NoDikes | 84.5 | None | None | Same as S1-D | HAT | Same |
| S2-D | G_Dikes / T_Dikes | 84.5 | PMF hydrograph, 5-min, 48 h | PMF local runoff | On; WS elev 96.7 m; 86 m; 0.1 h | HAT | 0 to 60 h |
| S2-N | G_NoDikes / T_NoDikes | 84.5 | PMF | PMF | Same as S2-D | HAT | 0 to 60 h |
| S3-D | G_Dikes / T_Dikes | 84.5 | PMF | PMF | Off | HAT | 0 to 60 h; run first; checks before S1/S2 |
| S3-N | G_NoDikes / T_NoDikes | 84.5 | PMF | PMF | Off | HAT | 0 to 60 h |

Table 8 - Water levels.

| Level | m a.s.l. | Used as |
| --- | --- | --- |
| Wadi bed at the axis; breach bottom | 23.0 | Final bottom elevation |
| Bottom outlet intakes | 36 and 44 | Closed; SP1 starts the pool at 46 |
| FSL = ogee crest | 84.5 | Initial WSE of every run; spillway weir elevation |
| Non-overflow crest | 95.5 | Weir elevation of the blocks |
| Parapet top and pier top | 96.7 | Pier elevation; SPW1 uses it for the blocks too |
| PMF pool (to be confirmed: 96.0 in the design table) | 96.7 | Breach trigger in S2; check level of S3 ± 0.2 m |
| Sea: highest astronomical tide (HAT) | from tide tables | Sea stage, all base runs |
| Sea: mean sea level | 0.0 | SS1 |
| Spillway capacity | 17,000 m³/s | S3-D discharge at the PMF pool ± 5 % |

Table 9 - Sensitivity runs.

| ID | Copy of | Change | Value |
| --- | --- | --- | --- |
| SW1 | S1-D, S2-D | Final bottom width | 43 m |
| SW2 | S1-D, S2-D | Final bottom width | 129 m |
| SW3 | S1-D, S2-D | Final bottom width | 156 m |
| ST1 | S1-D, S2-D | Formation time | 0.3 h |
| ST2 | S1-D, S2-D | Formation time | 0.05 h |
| SC1 | S1-D | Breach weir coefficient | 1.44 |
| SN1 | S1-D, S2-D | Manning table | n low column |
| SN2 | S1-D, S2-D | Manning table | n high column |
| SM1 | S1-D | Mesh | Half the cell size at the dam and in the town |
| SS1 | S2-D | Sea stage | Mean sea level 0.0 |
| SP1 | S1-D | Reservoir initial WSE | 46 m |
| SF1 | S2-D | Trigger | Set time = time of max pool in S3-D + 2 h |
| SI1 | S2-D | Inflow | 10,000-year hydrograph |
| SR1 | S1-D | Reservoir | Storage area with the elevation-volume curve instead of the 2D area |
| SPW1 | S3-D | Weir profile | Non-overflow blocks at 96.7 m (parapet credited) |

## Computation, outputs and checks

Table 10 - Computation and output settings.

| Where | Setting | Value |
| --- | --- | --- |
| Unsteady Flow > Boundary Conditions | Reservoir BC line | Flow hydrograph: PMF (S2, S3), 10,000-yr (SI1), none (S1) |
|  | Tributary BC lines J10, J11, J12 | Flow hydrographs from HEC-HMS (PMF local runoff); none in S1 |
|  | Sea BC line | Stage hydrograph, constant HAT (SS1: 0.0) |
|  | Open land edges | Normal depth, friction slope from the terrain at the edge |
| Unsteady Flow > Initial Conditions | Reservoir 2D area | Initial WSE 84.5 m (SP1: 46 m); downstream area dry |
| Plan > Computation Options > 2D | Equation set | Shallow Water Equations, Eulerian-Eulerian (SWE-EM) |
|  | Theta (implicit weighting) | 1.0 |
|  | Water surface tolerance / volume tolerance | 0.003 m / 0.003 m |
|  | Maximum iterations | 20 |
|  | Turbulence | Conservative, default coefficients |
| Plan > Advanced Time Step | Time step | Adaptive on Courant: max 1.0, halve above; min 0.5, double below; initial 1 s |
| Plan > Simulation window | S1 | 0 to 14 h (breach at 2 h) |
|  | S2, S3 | 0 to 60 h |
| Plan > Output | Hydrograph output interval | 10 s |
|  | Mapping output interval | 1 min for 2 h after the breach, then 5 min |
|  | Detailed output; restart file | On; write restart at 2 h in S1-D and reuse for S1 sensitivity runs |
| RAS Mapper > Results | Export | Max depth, max velocity, max D×V, arrival time (0.3 m), duration (> 0.3 m), hazard AIDR; GeoTIFF 2 m, EPSG:32640 |

Table 11 - Checks.

| Check | Pass if | Run |
| --- | --- | --- |
| Reservoir volume | 2D area volume at 84.5 m within 2 % of the elevation-volume curve | Before any run |
| Spillway rating | S3-D discharge at the PMF pool within 5 % of 17,000 m³/s | S3-D |
| Routed pool | S3-D peak pool within 0.2 m of the design team's value | S3-D |
| Volume error | Below 1 % | Every run |
| Stability | No oscillation in the breach hydrograph; no iteration warnings at the peak; smooth arrival-time map | Every run |
| Time step | Result unchanged when max Courant is halved | S1-D, S2-D |
| Mesh | Peak depth at the town entry changes less than 5 % in SM1 | SM1 |
| Peak outflow | Between the Ritter and the weir hand values: S1 base 38,000 to 71,000; S2 base 50,000 to 93,000 m³/s, or explained | Every failure run |
| Water reaches the sea | No volume lost at open edges; sea outflow plus storage equals inflow plus breach volume | Every run |
| Counts | People and plot counts reproduced by an independent QGIS overlay | Base runs |